SECTION 057300 DECORATIVE METAL RAILINGS WITH GLASS INFILL

GENERAL

RELATED DOCUMENTS

Drawings and general provisions of the Contract, including General and Supplementary Conditions and Division 1 Specification Sections, apply to this Section.

SUMMARY

This Section includes the following:

          - Component aluminum railings

          - Infill system for component aluminum railings

PERFORMANCE REQUIREMENTS

All railings shall be supplied to conform to applicable sections of the following codes:

International Building Code

ADAAG

Structural Performance:  Provide railings capable of withstanding the effects of gravity loads and the following loads and stresses within limits and under conditions indicated:

Handrails:

Uniform load of 50 lbf/ft. applied in any direction.

Concentrated load of 200 lbf. applied in any direction.

Uniform and concentrated loads need not be assumed to act concurrently.

Top Rails of Guards:

Uniform load of 50 lbf/ft. applied in any direction.

Concentrated load of 200 lbf. applied in any direction.

Uniform and concentrated loads need not be assumed to act concurrently.

Infill Area of Guards:

Horizontal concentrated load of 50 lbf. applied to 1 sq. ft. at any point in system, including panels, intermediate rails, balusters, or other elements composing infill area.  Load on infill area need not be assumed to act concurrently with loads on top rails.

Thermal Movements:  Provide exterior railings that allow for thermal movements resulting from the following maximum change (range) in ambient and surface temperatures by preventing buckling, opening of joints, overstressing of components, failure of connections, and other detrimental effects.  Base engineering calculation on surface temperatures of materials due to both solar heat gain and nighttime-sky heat loss.

Temperature Change (Range):  120°F, ambient; 180°F, material surfaces.

Control of Corrosion:  Prevent galvanic action and other forms of corrosion by insulating metals and other materials from direct contact with incompatible materials.

SUBMITTALS

Product Data:  For the following:

Manufacturer's product lines of mechanically connected railings.

Grout, anchoring cement, and paint products.

Shop Drawings:  Include plans, elevations, sections, details, and attachments to other work.

For installed products indicated to comply with design loads, include structural analysis data signed and sealed by the qualified professional engineer responsible for their preparation.

Samples for Initial Selection:  For products involving selection of color, texture, or design.

Mill Certificates:  Signed by manufacturers of stainless-steel products certifying that products furnished comply with requirements.

Qualification Data:  For professional engineer.

Product Test Reports:  Based on evaluation of comprehensive tests performed by a qualified testing agency, according to ASTM E 935.

QUALITY ASSURANCE

Source Limitations:  Obtain each type of railing through one source from a single manufacturer.

Mock-up Panel:  one section of railing system for verification.

Approximate Size:  ¼ to ½ size, using full size components.

Approved mockups may become part of the completed work if undamaged at time of substantial completion.

Approval of mockups does not constitute approval of deviations from the contract documents unless such deviations are specifically approved by architect in writing.

PROJECT CONDITIONS

Field Measurements:  Verify actual locations of walls and other construction contiguous with railings by field measurements before fabrication and indicate measurements on Shop Drawings.

COORDINATION AND SCHEDULING

Coordinate installation of anchorages for railings.  Furnish setting drawings, templates, and directions for installing anchorages, including sleeves, concrete inserts, anchor bolts, and items with integral anchors, that are to be embedded in concrete or masonry.  Deliver such items to project site in time for installation.

Schedule installation so wall attachments are made only to completed walls.  Do not support railings temporarily by any means that do not satisfy structural performance requirements.

PRODUCTS

MANUFACTURERS

Basis-of-Design Railing Product:  Subject to compliance with requirements, provide Speed-Rail® for glass as manufactured and assembled by Hollaender® Manufacturing or an approved equivalent.

METALS, GENERAL

Metal Surfaces, General:  Provide materials with smooth surfaces, without seam marks, roller marks, rolled trade names, stains, discolorations, or blemishes.

Brackets, Flanges, and Anchors:  Cast or formed metal of same type of material and finish as supported rails, unless otherwise indicated.

ALUMINUM

Aluminum, General:  Provide alloy and temper recommended by aluminum producer and finisher for type of use and finish indicated, and with not less than the strength and durability properties of alloy and temper designated below for each aluminum form required.

Extruded Bars and Tubing:  ASTM B 221, Alloy 6063-T5/T52, 6005-T5.

Extruded Structural Pipe and Round Tubing:  ASTM B 429, Alloy 6061-T6.

- Provide 1 ½ in IPS, (1.90 in OD) Standard Weight (Schedule 40) pipe for rails, Schedule 80 for posts, unless otherwise indicated

Drawn Seamless Tubing:  ASTM B 210, Alloy 6063-T832

Plate and Sheet:  ASTM B 209, Alloy 6061-T6

Die and Hand Forgings:  ASTM B 247, Alloy 6061-T6

Castings:  ASTM B 26/B 26M, Alloy Almag 535

Panel Clips:  Alloy 6063-T6.

STEEL

Perforated Sheet:  ASTM A1008.

FASTENERS

General:  Provide the following:

Aluminum Railings:  Alloy steel fasteners with JS-600 zinc plating.

Fasteners for Anchoring Railings to Other Construction:  Select fasteners of type, grade, and class required to produce connections suitable for anchoring railings to other types of construction indicated and capable of withstanding design loads.

Fasteners for Interconnecting Railing Components:

Provide Speed-Rail® structural slip-on fittings for interconnecting railing components and for attaching them to other work, unless exposed fasteners are unavoidable or are the standard fastening method for railings indicated. Systems using adhesive or pop rivet attachment systems will not be accepted.

Provide internal/external, reverse knurl, cup point, hex head set screws. Material to be alloy steel with JS 600 zinc plating.

Glass panels to be secured to top and bottom rails using Hollaender® 144-8 two piece glass panel clips. 144-8 clips will be secured to the rails using anodized aluminum tubular rivet nuts, size 5/16 – 18 UNC, and stainless steel socket head cap screws.

Anchors:  Provide concrete adhesive anchors where indicated or necessary.

GLASS INFILL PANELS FOR RAILINGS

Horizontal concentrated load of 50 lbf. applied to 1 sq. ft. at any point in system, including panels, intermediate rails, balusters, or other elements composing infill area.  Load on infill area need not be assumed to act concurrently with loads on top rails.

- For states on IBC 2009/2012:

- Tempered Glass: ASTM C1048, Fully Tempered, Condition A, Type 1 (Transparent Flat Glass), Quality Q3. Products shall comply with properties indicated for class, thickness, and manufacturing process that have been tested for surface and edge compression according to ASTM C1048 and for impact strength according to 16 CFR 1201 for Category 2 materials.

- Glass infill panel to be 3/8” thickness with maximum spacing between posts to be 4 ft.

- Glass to be monolithic or laminated tempered.

- ¨For states on IBC 2015 (Laminated glass is required):

- Tempered Glass: ASTM C1048, Fully Tempered, Condition A, Type 1 (Transparent Flat Glass), Quality Q3. Products shall comply with properties indicated for class, thickness, and manufacturing process that have been tested for surface and edge compression according to ASTM C1048 and for impact strength according to 16 CFR 1201 for Category 2 materials.

- Glass infill panel to be 3/16” lite, .060” lamination, 3/16” lite for a total thickness of 7/16” with maximum spacing between posts to be 4 ft.

- Glass to be laminated tempered.

- ¨ Resin panels to be acrylic 3/8” thickness, from Lumicor® or other architect approved vendor.

Handrail for ADA applications or stairs (as required)

          - Ramps that have a drop off of 30 inches or more on the side require guardrail, per above spec. Ramps with a rise greater than 6 inches shall have handrails on both sides.

          - Stairways shall have handrails on both sides.

          - Handrail will be attached to the guardrail sections using Hollaender model 85 adjustable brackets.

          - Handrail will be installed at a height of 34–38 inches above ramp surface or stair tread nosings.

          - Handrail will be anodized aluminum 6063 Sch. 40, 1 ½ in IPS nominal (1.90 in. OD) and shall have a continuous surface. Where necessary, lengths of the handrail will be spliced using Hollaender Model 70ES-8 internal locking splices.

          - Handrails shall return to a wall, guard or walking surface. If returning to the guard, Hollaender model 185 post return swivel shall be used to connect the end of the handrail to the guardrail post.

MISCELLANEOUS MATERIALS

Bituminous Paint:  Cold-applied asphalt emulsion complying with ASTM D 1187.

FABRICATION

General:  Fabricate railings to comply with requirements indicated for design, dimensions, member sizes and spacing, details, finish, and anchorage, but not less than that required to support structural loads.

Assemble railings in the shop to greatest extent possible to minimize field splicing and assembly.  Disassemble units only as necessary for shipping and handling limitations.  Clearly mark units for reassembly and coordinated installation.  Use connections that maintain structural value of joined pieces.

Cut, drill, and punch metals cleanly and accurately.  Remove burrs and ease edges to a radius of approximately 1/32 inch, unless otherwise indicated.  Remove sharp or rough areas on exposed surfaces.

Form work true to line and level with accurate angles and surfaces.

Fabricate connections that will be exposed to weather in a manner to exclude water.  Provide weep holes where water may accumulate.

Cut, reinforce, drill, and tap as indicated to receive finish hardware, screws, and similar items.

Connections:  Fabricate railings with non-welded connections, unless otherwise indicated.

Non-welded Connections:  Connect members with Speed-Rail® fittings of the structural slip-on type which fastens to the exterior of the pipe by means of an internal/external reverse knurl cup point set screw. Reverse knurl is required to ensure that screw does not come loose under vibration. Plain cup point screws will not be accepted.

Form changes in direction as follows:

By flush bends or by inserting prefabricated flush-elbow fittings.

Form simple and compound curves by bending members in jigs to produce uniform curvature for each repetitive configuration required; maintain cross section of member throughout entire bend without buckling, twisting, cracking, or otherwise deforming exposed surfaces of components.

Close exposed ends of railing members with prefabricated end fittings.

Provide wall returns at ends of wall-mounted handrails, unless otherwise indicated.  Close ends of returns unless clearance between end of rail and wall is 1/4 inch or less.

Brackets, Flanges, Fittings, and Anchors:  Provide wall brackets, flanges, miscellaneous fittings, and anchors to interconnect railing members to other work, unless otherwise indicated. Flanges to be sand cast from aluminum alloy 535 with anodized finish and fastened directly to the post by means of two reverse knurl cup point set screws.

Fabricate splice joints for field connection using an epoxy structural adhesive if this is manufacturer's standard splicing method.  Fabricate anchorage devices capable of withstanding loads imposed by railings.  Coordinate anchorage devices with supporting structure.

FINISHES, GENERAL

Comply with NAAMM's "Metal Finishes Manual for Architectural and Metal Products" for recommendations for applying and designating finishes.

Protect mechanical finishes on exposed surfaces from damage by applying a strippable, temporary protective covering before shipping.

Appearance of Finished Work:  Variations in appearance of abutting or adjacent pieces are acceptable if they are within one-half of the range of approved Samples.  Noticeable variations in the same piece are not acceptable.  Variations in appearance of other components are acceptable if they are within the range of approved Samples and are assembled or installed to minimize contrast.

ALUMINUM FINISHES

Finish designations prefixed by AA comply with the system established by the Aluminum Association for designating aluminum finishes.

Unless indicated otherwise, provide aluminum pipe with the following finish:

Anodized Finish:  AA-M10C22A41 (Architectural class, .7 mil thickness or greater)

EXECUTION

EXAMINATION

Examine plaster and gypsum board assemblies, where reinforced to receive anchors, to verify that locations of concealed reinforcements have been clearly marked for Installer.  Locate reinforcements and mark locations if not already done.

INSTALLATION, GENERAL

Fit exposed connections together to form tight, hairline joints.

Perform cutting, drilling, and fitting required for installing railings.  Set railings accurately in location, alignment, and elevation; measured from established lines and levels and free of rack.

Do not weld, cut, or abrade surfaces of railing components that have been coated or finished after fabrication and that are intended for field connection by mechanical or other means without further cutting or fitting.

Set posts plumb within a tolerance of 1/16 inch in 3 feet.

Align rails so variations from level for horizontal members and variations from parallel with rake of steps and ramps for sloping members do not exceed 1/4 inch in 12 feet.

Corrosion Protection:  Coat concealed surfaces of aluminum that will be in contact with grout, concrete, masonry, wood, or dissimilar metals, with a heavy coat of bituminous paint.

Adjust railings before anchoring to ensure matching alignment at abutting joints.

Fastening to In-Place Construction:  Use anchorage devices and fasteners where necessary for securing railings and for properly transferring loads to in-place construction.

RAILING CONNECTIONS

Non-welded Connections:  Use mechanical joints for permanently connecting railing components.  Use wood blocks and padding to prevent damage to railing members and fittings.

Expansion Joints:  Install expansion joints at locations indicated but not farther apart than required to accommodate thermal movement.  Provide slip-joint internal sleeve extending 2 inches beyond joint on either side, fasten internal sleeve securely to 1 side, and locate joint within 6 inches of post.

ANCHORING RAILING ENDS

Anchor railing ends to concrete and masonry with round flanges connected to railing ends and anchored to wall construction with anchors and bolts.

Anchor railing ends to metal surfaces with flanges bolted to metal surfaces and connected to railing ends using non-welded connections.

ATTACHING HANDRAILS TO WALLS

Attach handrails to wall with wall brackets.  Provide brackets with 1-1/2-inch clearance from inside face of handrail and finished wall surface.

Locate brackets as indicated or, if not indicated, at spacing required to support structural loads.

Secure wall brackets to building construction as indicated, or if not indicated, as follows:

For concrete and solid masonry anchorage, use drilled-in expansion shields and hanger or lag bolts.

For hollow masonry anchorage, use toggle bolts.

Provide blocking between studs in stud wall construction.

ADJUSTING AND CLEANING

Clean aluminum and stainless steel by washing thoroughly with clean water and soap and rinsing with clean water.

PROTECTION

Protect finishes of railings from damage during construction period with temporary protective coverings approved by railing manufacturer.  Remove protective coverings at time of Substantial Completion.

Restore finishes damaged during installation and construction period so no evidence remains of correction work.  Return items that cannot be refinished in the field to the shop; make required alterations and refinish entire unit, or provide new units.

END OF 057300 DECORATIVE METAL RAILINGS